Purpose

This lab is designed to provide real-world experience in setting up Cisco wireless access points and configuring wireless local area networks (WLANs) with a wired setup and multiple Service Set Identifiers (SSIDs).

Background Information

Think of a wireless access point (WAP) as a bridge between fully wired and wireless networks. It is a networking device designed to connect wireless devices to an existing wired network. Utilizing WAPs to connect all wireless-capable devices in a network is far more efficient than connecting each through wires and cabling.

To install an access point, it is first connected to a wired network made of routers or switches, typically through an Ethernet cable. Once connected, the AP broadcasts a wireless signal using a specific frequency (e.g., 2.4 GHz or 5 GHz) and Wi-Fi standard (e.g., 802.11ac). Then, the AP can be configured with its own Service Set Identifier, or SSID, which is the name of the wireless network that users see when connecting to Wi-Fi on their devices. Now, data can be transmitted from the wired network to wireless devices connected to the AP. APs can also be configured with many security settings, such as passwords and encryption to prevent unwanted access. Clients within the AP’s range can easily detect its SSID and use its credentials to connect to it.

Cisco allows additional configurations to be through a wireless LAN controller (WLC); it can be accessed either through the web-based GUI, through inputting the WLC’s IP address into a web browser, or the command-line interface (CLI). Using a centralized management tool like a WLC is extremely helpful as it can simplify the AP configuration process and provide better control over networks.

This lab required the use of previous knowledge of IP Routing learned through the CCNA course, such as NAT and router-on-a-stick configurations. Configuring NAT (Network Address Translation) allows private IP addresses to be translated to a public IP address to access the Internet. “Router on a stick” configuration is used to route traffic between multiple VLANs on a network by creating multiple sub interfaces on one physical router port, which allows for traffic from different VLANs to be routed through a single port as though there are multiple ports in its place. These concepts were implemented in this lab to route traffic through each device.

A portion of this lab required installing a RADIUS (Remote Authentication Dial-In User Service) server. RADIUS servers provide users with a centralized way to manage authentication, authorization, and accounting and is commonly used in wireless networks. RADIUS servers receive authentication requests from clients, such as access points, VPNs, etc., and either grants or denies access. Such technology is often deployed in enterprise network infrastructure in the real world.

Lab Summary

To complete this lab, we followed these procedures:

- Designed a topology with a TFTP server, a PoE (Power over Ethernet) Switch, wireless devices, and an access point.

- Connected the access point to the computer with a console cable and an Ethernet cable.

- Consoled into the switch and set up a TFTP server.

- Set the IP addresses for the access point and the computer.

- Entered the AP’s IP address in a web browser (Microsoft Edge preferred) to access the GUI.

- Using the GUI, set up three SSIDs for VLANs 2, 3, and 4.

- Set up WPA pre-shared key authentication on the access point.

- Set VLAN configurations on the switch.

- Configured NAT on router interfaces.

- Configured a router-on-a-stick set-up.

- Troubleshooted network until functioning.

Lab Commands

ip routing
Enables IP routing in switch configuration mode.

lwapp ap ip address [ipv4 address] [subnet mask]
Sets the IPv4 address for the access point.

lwapp ap ip default-gateway [ipv4 address]
Sets the default gateway address for the access point.

archive download-sw /overwrite /reload [file]
Overwrites and reloads a new file, in this case, TFTP file for the server.

ip nat pool [pool name] [starting IP] [ending IP] netmask [subnet mask]
Defines a range of IP addresses to be used with NAT configurations.

ip dhcp pool [vlan name]
Creates a new DHCP pool allowing to make parameters for a specific VLAN. 

network [ipv4 address] [subnet mask]
Sets the range of addresses to be assigned during use of DHCP.

ip dhcp excluded-address [ipv4 address]
Specifies addresses to not be assigned by the DHCP server.

switchport mode trunk
Configures a switch interface as a trunk port, allowing traffic from multiple VLANs over a single physical port.

ip route [destination network] [subnet mask] [next-hop ipv4 address] [metric number]
Creates a static route to define the paths a packet should follow in order to reach the destination network. Used in situations where no routing protocols are used.

dot11 ssid [SSID name]
Used on wireless access point to set the SSID name.

authentication open
Allows clients to associate with the SSID with open authentication

authentication key-management wpa version 2
Specifies the key management method for securing wireless communications, specifically WPA2 (Wi-Fi Protected Access version 2), which provides high security.

Topology & IP Scheme

Configurations

Router1:

Show run:

Building configuration...
Current configuration : 1602 bytes
version 15.5
service timestamps debug datetime msec
service timestamps log datetime msec
no platform punt-keepalive disable-kernel-core
hostname Router
boot-start-marker
boot-end-marker
vrf definition Mgmt-intf
address-family ipv4
exit-address-family
address-family ipv6
exit-address-family
no aaa new-model
subscriber templating
multilink bundle-name authenticated
license udi pid ISR4321/K9 sn FDO21491LXV
license accept end user agreement
license boot level securityk9
spanning-tree extend system-id
redundancy
mode none
vlan internal allocation policy ascending
interface GigabitEthernet0/0/0
ip address 10.0.0.193 255.255.255.0
ip nat inside
negotiation auto
interface GigabitEthernet0/0/1
ip address dhcp
ip nat outside
negotiation auto
interface Serial0/1/0
interface Serial0/1/1
interface GigabitEthernet0
vrf forwarding Mgmt-intf
no ip address
shutdown
negotiation auto
interface Vlan1
no ip address
shutdown
ip nat pool AP_POOL 192.168.40.2 192.168.41.254 netmask 255.255.254.0
ip nat inside source list 1 pool AP_POOL overload
ip forward-protocol nd
no ip http server
no ip http secure-server
ip tftp source-interface GigabitEthernet0
ip route 0.0.0.0 0.0.0.0 192.168.40.1
access-list 1 permit 10.0.0.0 0.0.0.255
control-plane
line con 0
exec-timeout 0 0
stopbits 1
line aux 0
stopbits 1
line vty 0 4
login
ntp server pool.ntp.org
end

Switch:

Show run:

Building configuration...

Current configuration : 2322 bytes

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

hostname Switch

boot-start-marker

boot-end-marker

no aaa new-model

system mtu routing 1500

authentication mac-move permit

ip subnet-zero

ip routing

ip dhcp excluded-address 10.0.0.65

ip dhcp excluded-address 10.0.0.129

ip dhcp excluded-address 10.0.0.1

ip dhcp excluded-address 10.0.0.193

ip dhcp pool VLAN2

network 10.0.0.0 255.255.255.192

default-router 10.0.0.1

domain-name cisco.com

dns-server 1.1.1.1

ip dhcp pool VLAN3

network 10.0.0.64 255.255.255.192

default-router 10.0.0.65

dns-server 1.1.1.1

domain-name cisco2.com

ip dhcp pool VLAN4

network 10.0.0.128 255.255.255.192

default-router 10.0.0.129

dns-server 1.1.1.1

domain-name cisco3.com

spanning-tree mode pvst

spanning-tree etherchannel guard misconfig

spanning-tree extend system-id

vlan internal allocation policy ascending

interface FastEthernet0/1

interface FastEthernet0/2

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 2

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/4

interface FastEthernet0/5

interface FastEthernet0/6

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

interface FastEthernet0/23

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

ip address 10.0.0.252 255.255.255.192

interface Vlan2

ip address 10.0.0.1 255.255.255.192

interface Vlan3

ip address 10.0.0.65 255.255.255.192

interface Vlan4

ip address 10.0.0.129 255.255.255.192

ip default-gateway 10.0.0.193

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.193

ip http server

ip sla enable reaction-alerts

line con 0

line vty 0 4

login

line vty 5 15

login

end

Access Point:

Show run:

Building configuration...
Current configuration : 4735 bytes
version 15.3
no service pad
service timestamps debug datetime msec
service timestamps log datetime msec
service password-encryption
hostname ap
logging rate-limit console 9
enable secret 5 $1$iqK4$gs1Txjbm6FNlnfh/zkixY0
aaa new-model
aaa group server radius rad_eap
server name freeradius
aaa group server radius rad_mac
aaa group server radius rad_acct
aaa group server radius rad_admin
aaa group server tacacs+ tac_admin
aaa group server radius rad_pmip
aaa group server radius dummy
aaa group server radius rad_eap3
server name freeradius
aaa authentication login eap_methods group rad_eap
aaa authentication login mac_methods local
aaa authorization exec default local
aaa accounting network acct_methods start-stop group rad_acct
aaa session-id common
no ip source-route
no ip cef
dot11 pause-time 100
dot11 syslog
dot11 ssid CCNP_P1_0507_VLAN2
   vlan 2
   authentication open
   authentication key-management wpa version 2
   mbssid guest-mode
   wpa-psk ascii 7 112A1016141D5A5E57
dot11 ssid CCNP_P1_0507_VLAN3
   vlan 3
   authentication open
   authentication key-management wpa version 2
   mbssid guest-mode
   wpa-psk ascii 7 112A1016141D5A5E57
dot11 ssid CCNP_P1_0507_VLAN4
   vlan 4
   authentication open eap eap_methods3
   authentication network-eap eap_methods3
   authentication key-management wpa
   mbssid guest-mode
dot11 wpa handshake timeout 500
no ipv6 cef
username Cisco privilege 15 password 7 02250D480809
bridge irb
interface Dot11Radio0
no ip address
shutdown
encryption vlan 3 mode ciphers aes-ccm
antenna gain 0
station-role root
bridge-group 1
bridge-group 1 subscriber-loop-control
bridge-group 1 spanning-disabled
bridge-group 1 block-unknown-source
no bridge-group 1 source-learning
no bridge-group 1 unicast-flooding
interface Dot11Radio1
no ip address
encryption vlan 3 mode ciphers aes-ccm
encryption vlan 4 mode ciphers aes-ccm
encryption vlan 1 key 2 size 128bit 7 F820D71D63394A6BD63DE57FD26B transmit-key
encryption vlan 1 mode wep mandatory
encryption vlan 2 mode ciphers aes-ccm
ssid CCNP_P1_0507_VLAN2
ssid CCNP_P1_0507_VLAN3
ssid CCNP_P1_0507_VLAN4
antenna gain 0
peakdetect
dfs band 3 block
mbssid
channel dfs
station-role root
interface Dot11Radio1.1
encapsulation dot1Q 1 native
bridge-group 1
bridge-group 1 subscriber-loop-control
bridge-group 1 spanning-disabled
bridge-group 1 block-unknown-source
no bridge-group 1 source-learning
no bridge-group 1 unicast-flooding
interface Dot11Radio1.2
encapsulation dot1Q 2
bridge-group 2
bridge-group 2 subscriber-loop-control
bridge-group 2 spanning-disabled
bridge-group 2 block-unknown-source
no bridge-group 2 source-learning
no bridge-group 2 unicast-flooding
interface Dot11Radio1.3
encapsulation dot1Q 3
bridge-group 3
bridge-group 3 subscriber-loop-control
bridge-group 3 spanning-disabled
bridge-group 3 block-unknown-source
no bridge-group 3 source-learning
no bridge-group 3 unicast-flooding
interface Dot11Radio1.4
encapsulation dot1Q 4
bridge-group 4
bridge-group 4 subscriber-loop-control
bridge-group 4 spanning-disabled
bridge-group 4 block-unknown-source
no bridge-group 4 source-learning
no bridge-group 4 unicast-flooding
interface Dot11Radio1.21
interface Dot11Radio1.31
interface Dot11Radio1.41
interface GigabitEthernet0
no ip address
duplex auto
speed auto
interface GigabitEthernet0.1
encapsulation dot1Q 1 native
bridge-group 1
bridge-group 1 spanning-disabled
no bridge-group 1 source-learning
interface GigabitEthernet0.2
encapsulation dot1Q 2
bridge-group 2
bridge-group 2 spanning-disabled
no bridge-group 2 source-learning
interface GigabitEthernet0.3
encapsulation dot1Q 3
bridge-group 3
bridge-group 3 spanning-disabled
no bridge-group 3 source-learning
interface GigabitEthernet0.4
encapsulation dot1Q 4
bridge-group 4
bridge-group 4 spanning-disabled
no bridge-group 4 source-learning
interface GigabitEthernet0.21
interface GigabitEthernet0.31
interface GigabitEthernet0.41
interface BVI1
description freaky
mac-address 44d3.ca19.96af
ip address 10.0.0.253 255.255.255.0
ipv6 address dhcp
ipv6 address autoconfig
ipv6 enable
ip default-gateway 192.168.40.1
ip forward-protocol nd
ip http server
no ip http secure-server
ip http help-path http://www.cisco.com/warp/public/779/smbiz/prodconfig/help/eag
ip radius source-interface BVI1
radius-server attribute 32 include-in-access-req format %h
radius server freeradius
address ipv4 10.0.0.254 auth-port 1645 acct-port 1646
key 7 123A0C041104
bridge 1 route ip
line con 0
line vty 0 4
transport input all
end

Problems

At times, when we resumed the lab the next day, we weren’t able to connect to the GUI during class. The cause of this wasn’t obvious and was primarily solved by simply reopening the lab the following day.

After troubleshooting and verifying our configurations were functioning, we discovered that the problem related to Layer 1, since the Ethernet cable connecting the switch to the router was physically connected to the wrong ports. To fix this, the cable was switched from port g0/0/0 to g0/0/1.

When initially configuring the switch, we didn’t know it was required for each VLAN to have a name. We quickly resolved this by using the  vlan name [name] command.

After being unable to connect to our network, we troubleshooted with a set of pings on each device and discovered that the switch could not ping the access point. Entering the show ip interface brief command showed IP address configurations and allowed us to figure out that VLAN 1 was missing an IP address. Once the VLAN interface was up, by using the no shutdown command, and had an IP, pings were able to travel through the switch.

Conclusion
Completing this lab helped us experience working with wireless devices along with networking practices usually used with wired topologies. We learned how to set up a Cisco LWAP and use multiple SSIDs to communicate between devices.

Lab Signoff

s